FloodGuard AI

Adaptive Hyper-Local Flash Flood Early Warning & Decision Support System

SIH26192 — Flash Flood Prediction System for Hilly Regions using Multi-Source Data

SIH Technical / Detailed Project Report

# 1. Executive Summary

FloodGuard AI is an adaptive, hyper-local flash flood early warning and decision-support system designed for hilly and vulnerable regions of India. The system combines rainfall, water-level/flow, soil saturation, weather forecasts, terrain and historical flood information to estimate localized flash-flood risk.

Instead of relying on a single rainfall threshold or a single sensor, the proposed platform combines multiple evidence sources and produces a location-specific risk level, confidence score, reasons for the prediction, and actionable alerts. The system is designed around short lead-time forecasting: a 0–6 hour nowcast and a 6–24 hour forecast, with the architecture capable of extending the outlook when reliable forecast data is available.

# 2. Problem Statement

Flash floods in hilly regions can develop rapidly after intense or prolonged rainfall. Terrain, drainage characteristics, soil saturation, river/stream response and changing weather conditions can cause the same rainfall amount to produce very different flood impacts at different locations.

The key challenge is therefore not only to detect rainfall, but to answer:
• Where is flooding likely to occur?
• How severe is the current risk?
• How soon could conditions deteriorate?
• How confident is the prediction?
• Which roads, villages or exposed areas need attention?
• What action should authorities and communities consider?

# 3. Existing System Gap

Existing approaches can provide rainfall forecasts, river-level monitoring, weather warnings, flood maps or sensor-based alerts, but these capabilities are often fragmented. A rainfall warning may not directly indicate which village or road segment is at risk, while a sensor-only system can be limited by sparse coverage, sensor failure or communication outages.

FloodGuard AI addresses this engineering gap by integrating multiple data sources into one adaptive risk pipeline and converting the resulting prediction into a hyper-local map and actionable warning.

# 4. Proposed Solution

The proposed system follows a multi-layer architecture:

1. Historical + GIS layer — historical flood events, terrain/elevation, drainage and other spatial information establish the baseline susceptibility.
2. Real-time monitoring — rainfall, water level/flow and soil saturation provide current ground conditions.
3. Weather intelligence — forecast rainfall is incorporated to anticipate near-future conditions.
4. Data validation — abnormal, missing or unreliable sensor readings are detected so one faulty input does not dominate the prediction.
5. Adaptive risk engine — machine learning, local thresholds and multi-source evidence are combined.
6. Hyper-local risk mapping — risk is represented at village/ward/road/stream-segment level where data permits.
7. Confidence + explanation — the system reports why risk increased and how confident the estimate is.
8. Decision + alert layer — authorities receive actionable alerts and communities can receive warning and evacuation information.

# 5. Core Innovation

The innovation is not simply “AI predicts floods.” The engineering contribution is the integration of adaptive, hyper-local and uncertainty-aware flood prediction with an action layer.

Key innovations:
• Hyper-local risk instead of only district-level warning.
• Adaptive local thresholds instead of one universal rainfall threshold.
• Multi-source fusion of rainfall, water level, soil saturation, weather forecast, terrain and historical data.
• Confidence score and prediction reasons alongside risk.
• Separate current-condition nowcast from future forecast.
• Risk-to-action conversion for road closure, inspection, evacuation preparation and emergency response.
• Graceful handling of missing or unreliable sensor data.
• Support for low-connectivity operation through local/edge processing in advanced deployment.

# 6. System Architecture

DATA SOURCES
    ↓
Historical Flood Data + GIS/Terrain + Weather Forecast + Rainfall + Water Level/Flow + Soil Saturation
    ↓
DATA INGESTION & PREPROCESSING
Cleaning • Integration • Feature Extraction • Missing Data Handling
    ↓
REAL-TIME MONITORING + DATA VALIDATION
Sensor Health • Anomaly Detection • Reliability Scoring
    ↓
ADAPTIVE RISK ENGINE
ML Model + Local Thresholds + Forecast Rainfall + Current Conditions
    ↓
0–6 HOUR NOWCAST  |  6–24 HOUR FORECAST
    ↓
HYPER-LOCAL FLOOD RISK MAP
Location • Risk • Confidence • Reasons • Exposure
    ↓
DECISION & ALERT ENGINE
Warning • Road/route status • Evacuation support • Authority response
    ↓
DASHBOARD + SMS/APP/LOCAL ALERTS

# 7. Data Inputs

Primary data inputs:
• Rainfall intensity and cumulative rainfall
• River/stream water level or flow
• Soil moisture / soil saturation
• Weather forecast
• Elevation and terrain characteristics
• Drainage/stream network information
• Historical flood events and impact information
• Satellite/remote-sensing information where available
• Population, roads and critical infrastructure for exposure analysis

The exact sensor/data mix can be scaled according to the deployment area and prototype budget.

# 8. Adaptive Rainfall Intelligence

A single rainfall threshold is not sufficient for every location. FloodGuard AI considers short-term and cumulative rainfall along with antecedent wetness.

Example rainfall windows:
• 1 hour
• 3 hours
• 6 hours
• 12 hours
• 24 hours
• Multi-day cumulative rainfall

The local threshold can be calibrated using location, season, terrain/drainage characteristics, soil saturation and historical flood events. Threshold values shown in a prototype should be treated as engineering examples until validated with local data.

# 9. Machine Learning / Risk Modelling

For the prototype, a practical tree-based model such as Random Forest or XGBoost can be used because it works well with structured/tabular multi-source features and is comparatively explainable.

Example feature groups:
• Rainfall intensity and cumulative rainfall
• Forecast rainfall
• Water level/flow trend
• Soil saturation
• Terrain/elevation/drainage indicators
• Historical flood susceptibility
• Seasonal conditions

The model produces a localized risk estimate rather than claiming an unsupported exact probability of flooding. The final operational risk state should be generated using multiple evidence sources and validation rules.

# 10. Risk Classification

A prototype can display four operational levels:

LOW — routine monitoring
MODERATE — increased monitoring and preparedness
HIGH — proactive authority action and public warning as appropriate
CRITICAL — urgent response/evacuation procedures according to validated local disaster-management protocols

The exact thresholds should be calibrated and approved for the deployment location.

# 11. Confidence and Explainability

FloodGuard AI reports risk together with confidence and reasons. For example:

Risk: HIGH
Confidence: HIGH
Reasons:
• Heavy recent rainfall
• Forecast rainfall increasing
• Water level rising
• Soil saturation high
• Historical susceptibility elevated

If important sensors are unavailable or models disagree, confidence is reduced instead of presenting false precision.

# 12. Hyper-Local Risk Map

The dashboard can divide the target area into small spatial units or meaningful segments such as villages, wards, roads, drainage/stream segments or other locally useful zones.

Each zone can contain:
• Location / cell ID
• Current rainfall
• Water level/flow
• Soil saturation
• Risk level
• Confidence
• Exposure
• Prediction time window
• Main reasons for risk

This allows authorities to focus on specific high-risk locations rather than issuing a blanket warning across a large district.

# 13. Decision Support and Alerts

The system converts prediction into operational information.

For authorities:
• Identify high-risk villages/roads
• Prioritize field inspection
• Prepare evacuation resources
• Consider road/route restrictions
• View affected population and critical infrastructure
• Monitor sensor health

For communities:
• Current local risk
• Expected risk window
• Recommended safety action
• Safe-zone/evacuation information where configured
• Local-language and low-connectivity alerts in advanced deployment

The platform supports decisions; official warnings and evacuation instructions remain under the responsible authorities and validated disaster-management procedures.

# 14. Prototype / Dashboard

The proposed prototype contains:
• Dashboard
• Prediction
• Hyper-local Risk Map
• Emergency Alerts
• Evacuation Support
• About / Technology

Demo inputs:
Rainfall + Water Level + Soil Saturation

Demo outputs:
Risk Percentage/Level + Confidence + Reasons + Map Location + Alert/Action

A representative hilly-region demo can use a sample location such as Chamoli, Uttarakhand, without implying that the prototype is an operational warning service for that location.

# 15. Technology Stack

Frontend:
• React
• Leaflet / MapLibre for maps

Backend:
• FastAPI or Node.js
• REST APIs

Database:
• PostgreSQL + PostGIS
• Time-series storage where required

ML / Analytics:
• Python
• scikit-learn
• XGBoost / LightGBM

GIS / Spatial:
• QGIS
• GDAL
• Rasterio
• GeoPandas

IoT / Sensors:
• ESP32
• Rainfall sensor/gauge
• Water-level sensor
• Soil-moisture sensor
• LoRa/LoRaWAN or GSM/4G for communication

Deployment:
• Cloud dashboard
• Optional Raspberry Pi/edge gateway for local processing and resilience

# 16. Methodology & Process of Implementation

01 — Data Collection

Collect historical flood, rainfall, terrain, weather and monitoring data.

02 — Real-Time Monitoring

Receive rainfall, water-level/flow and soil-saturation readings.

03 — Data Processing

Clean, integrate and validate incoming data; handle missing and anomalous values.

04 — Risk Modelling

Combine ML prediction, adaptive thresholds, current conditions and forecast information.

05 — Hyper-Local Mapping

Generate location-specific risk and confidence.

06 — Decision & Alerts

Translate risk into authority actions and community warnings.

07 — Feedback & Calibration

Use new events and validated observations to improve local calibration and monitor model drift.

# 17. Handling Sensor / Internet Failure

The system should degrade gracefully.

If a sensor becomes unavailable:
• Continue prediction using available rainfall, water-level, soil, weather, historical and spatial information.
• Reduce confidence when important measurements are missing.
• Flag the sensor as unavailable.

If internet connectivity fails in an advanced deployment:
• Local/edge processing can continue using recent data and configured rules.
• Local alarms can operate where hardware is available.
• Buffered observations can synchronize when connectivity returns.

# 18. Feasibility and Viability

Feasibility:
• Uses established technologies such as IoT sensors, GIS, weather data, ML and web dashboards.
• Prototype can begin with a limited sensor set and a selected hilly-region study area.
• Architecture is modular and can scale from demonstration to wider deployment.
• Software components can be built using widely available open-source technologies.

Viability:
• Provides a single decision-support interface instead of fragmented monitoring.
• Hyper-local warnings can help focus limited emergency resources.
• Confidence and sensor-health information improve transparency.
• The same architecture can be calibrated for different regions.

Cost and operational feasibility depend on sensor density, communication infrastructure, data availability and deployment scale.

# 19. Expected Outcomes / Impact

- Earlier identification of rapidly changing flood risk

- More localized warnings

- Better prioritization of vulnerable roads, villages and critical infrastructure

- Reduced dependence on a single sensor or rainfall threshold

- More transparent AI-assisted decisions through reasons and confidence

- Better coordination between monitoring, warning and response

- Scalable architecture for other flood-prone hilly regions

- Potential to reduce avoidable response delays and improve preparedness

# 20. Future Scope

- More satellite-derived rainfall and soil-moisture inputs

- Streamflow forecasting

- Advanced spatiotemporal ML models

- Automated safe-route and evacuation-route optimization

- Local-language voice/SMS alerts

- More robust edge/offline operation

- Dynamic sensor placement based on risk and uncertainty

- Continuous model-drift monitoring and local recalibration

- Integration with official disaster-management systems

# 21. Conclusion

FloodGuard AI is designed as an end-to-end flash-flood early warning and decision-support platform. Its central approach is to combine multi-source observations, adaptive local thresholds, machine learning, weather forecasts and hyper-local mapping instead of depending on a single indicator.

The system is intended to answer five operational questions: WHERE is the risk, WHAT is happening now, WHAT may happen next, HOW confident is the estimate, and WHAT action should be considered. This makes the proposed solution suitable as a practical SIH prototype while leaving operational thresholds and official warning decisions to local validation and responsible authorities.

# 22. Project Information

Problem Statement ID: SIH26192
Problem Statement Title: Flash Flood Prediction System for Hilly Regions using Multi-Source Data
Proposed Solution: FloodGuard AI — Adaptive Hyper-Local Flash Flood Early Warning & Decision Support System
Theme/Domain: Disaster Management / Flood Early Warning
Prototype Focus: Hyper-local flash-flood prediction, risk mapping and actionable alerts